TX-Pro II AT/ATX System Board

Key Features

The key features of this mainboard include:

Socket-7 Processor Support

1	Supports all recent socket-7 processors including the Intel P55C (Pentium MMX), the Cyrix/IBM 6x86L/6x86MX /MII, the AMD K6/K6-2/K6-III, IDT C6, and WinChip 2/2A CPUs

1	Supports socket-7 processors with system bus frequencies

of 60/66/75 MHz

Supports CPU Plug and Play to automatically configure

the CPU

1	512K external Level 2 cache memory is provided onboard

Memory Support

Two DIMM slots for FP/EDO/SDRAM 168-pin memory modules

1	Maximum installed memory can be 2 x 256 MB = 512 MB

Expansion Slots

Three 32-bit PCI slot

Two 8/16-bit ISA slots

Onboard IDE channels

1	Primary and Secondary PCI IDE channels

Support for PIO (programmable input/output) modes

1	Support for Bus mastering and U1traDMA 33 modes

Power Supply and Power Management

Provides AT/ATX power connector

Support for Power button/Suspend Switch

1	Supports Wake on Modem, Wake on LAN and Wake on

Alarm

2

Key Features

Built-in Graphics System

1	Onboard 64-bit Graphics Accelerator

Shared memory architecture allows a maximum of 4 MB main

memory to act as frame buffer

Supports high resolutions up to 1024 x 768 pixels

Sound System

Meets PC98 audio specification

1 Full duplex playback and recording with built-in 16-bit

CODEC

1	HRTF 3D professional audio supports both Direct Sound

3D® and A3D© compatible interfaces plus support for 4

channel speakers

Drivers support Windows 3.1/95/98/NT 4.0

Built-in 32 ohm earphone buffer and 3D surround

Provides MPU-401 Game/MIDI port and legacy Sound

Blaster 16 support

Downloadable Wave-table Synthesizer supports Direct

Music©

1	Digital Audio Interface with 24-bit stereo, 44KHz

sampling rate and measured 120dB audio quality

Optional optic fibre interface which enables communication

with MiniDisk or high-end audio systems.

Stereo Mixer supports analog mixing from CD-Audio,

Line-In, and digital mixing from voice, FM/Wave-table and

digital CD-Audio

Onboard I/O Ports

1 Floppy disk drive port with 1Mb/s transfer rate

1 One serial port with 16550-compatible fast DART

1 One parallel port with support for ECP and EPP

1 Two USB ports & one PS/2 ports (optional)

One optional infrared port

3