1. General information

  - INTRODUCTION

The PM 5509 is a TV Service pattern generator for use in TV/Colour TV Service and Video equipment like VCR (video-cassette recorders). It features 10 test patterns 5 of them in colour, modulated on an RF carrier which can be tuned to practically all available TV channels including IF.

The tuning is carried out electronically. Two small meters on the front panel indicate the TV Band in use and the channel frequency.

Test patterns include a checker board, dots, cross hatch, grey scale and a simple circle pattern for the usual Black and White alignments. For colour a red pattern, a white pattern, a standard colour-bar with white reference and 1 special test pattern is available. This last pattern enables "on the screen" alignment of PAL-delay line and PAL demodulators. Furthermore the grayscale pattern is combined with definition-lines up to 5 MHz for checking video equipment e.g. video recorders. For the V.C.R., another special test pattern is used. An eight-step saturation Bar combined with a colour definition-line signal up to 1 MHz.

Chroma (including) Burst can be continuously adjusted from 0 - 100 % and the RF output amplitude is adjustable to at least 60 dB down, far enough to check the C.T.V. receivers sensitivity. Sound carrier of 5.5 MHz can be switched off (other frequencies e.g. 4.5 or 6.0 MHz can be chosen by changing a solder joint inside the instrument). It can also be modulated with the internal 1 KHz sinewave but external modulation allows the connection of e.g. a record-player or tape recorder. A special DIN-plug for this purpose is located at the rear side of the instrument.

For a.o. external triggering of an oscilloscope either a field or line-pulse is available. Video output signal is the standard 1 Volt in 75 ohms. The polarity of the video signal can be changed inside the instrument but is normally positive. External video-modulation is also possible so that one can use the PM5509 as a simple modulator for e.g. camera signals. For connection of a VCR there is a special DIN-plug available on the rear of the PM5509