12

| - "COLOUR BAR" | : 75 % contrast standard colour bar with reference white field in lower part of screen. |  |  |  |
| --- | --- | --- | --- | --- |
| Bar |  | Relative luminance amplitude | Chroma phase | Relative chroma amplitude |
| White |  | 0.75 | - | - |
| Yellow |  | 0.67 | 167° | ±0.33 |
| Cyan |  | 0.53 | 283° | ±0.47 |
| Green |  | 0.44 | 2410 | ±0.44 |
| Magenta |  | 0.31 | 61° | ±0.44 |
| Red |  | 0.23 | 103° | ±0.47 |
| Blue |  | 0.08 | 347° | ±0.33 |
| Black |  | 0 | - |  |

- "VCR"	: VCR pattern (R-Y signal) 8 sets of colour definition lines

ranging from 100 kHz - 1 MHz and with 8 steps of linear in

creasing saturation from 0 - 100 %

1.2.2. TV Systems

I

PM 5509-G	: 5.5 MHz (CCI R system G)

PM 5509-1	: 6 MHz (CCIR system I)

PM 5509-M	: 4.5 MHz (RTMA system M)

1.2.3. Video