P.O. Box 38-828

PETONE, Wellington

DATA SHEET	Phone - (64-4) 9393 777

FAX - (64-4) 9393 778

Date - 9th February, 2000	e-mail = sales@ casa.co.nz	Ref - RACK-SHL.FRP

or 19" RA C

This SHELF has been designed to provide 19" Rack mounting/fixing for FREE STANDING equipments such that many non-standard products can be installed in Standard 19" Equipment Racks. Typical equipment include Audio, Video, Industrial, Telecommunications, Data Systems, Modems, Batteries and other Electronic Apparatus not exceeding 437mm (17.2") wide.

Features Include:

1) Standard UNIT heights (44.45mm 1.75") from 2-4 units @ 200, 300 or 400mm deep (other sizes 350, 450 etc. to order)

2) Folds at rear and/or front can be applied as "Fold-Up" and/or "Fold Down" to give added stiffness and strength to shelves.

3) Simple Basic design permits standard & SPECIAL lengths, finishes and variants to be produced economically even in small

quantities with features including punching for ventilation and retrofittable / rear fixing for larger or heavier equipment.

4) Custom design can be undertaken for special requirements. Please enquire giving details of desired/required features,

facilities, and quantities etc. (a sketch or drawing will be appreciated). Many variations can be processed from "blanked" stock.

CASA's Design Philosophy: Compatability, Adaptability, Symplicity and Acceptability achieves sound Engineering, Cost

Economy and Aesthetics. 	NOTE - See reverse of this Data Sheet, or separate drawings, for basic dimensional information.

Specification Details:

Preferred / Standard Sizes:

Special Heights & Depths to order: (minimum quantity may apply) Cutting & Nominal Dimensional Tolerance:

2 Unit (88.1mm) Stock

-3 Units (132.5mm) Stock, -

4 Units (177mm) Stock

_

-------- 	--------------

1-10 Units (1 U=44.45mm)

+l- 0.25mm (to 500mm)

Powder Coat, Zinc Plated or Anodized (if Aluminum) 2 Unit = 3Kg (approx.)

Unit _ _3.5Kg (approx.)

4 Unit = 4Kg (appro# 	_

((depends upon depth etc.)

Finishes:

Material: 	Mild Steel (or Aluminum)

Part Number.- RS-xxUxYYzz=MAT=F adder/suffix

(full deburring of all cut, drilled, punched and tapped holes & edges is standard)

Weight:

(typical/nominal weights for 300mm deep using 2mm Mild Steel)

Please compose a

Part Number

according to this

system and use on

your official order.

RS - ,Prefix to indicate "RACK SHELF".

xxU - ,HEIGHT in RACK Units (multples of 44.45milimeters)-_ _	_

YY - ,Depth behind mounting face, in_'mm"-X200, 300, 400 others made to orders .

zz - _ Fold designations (F = Rear Fold UP, FF = Front & Rear Folded UP etc.)-	_

MAT - Material used MS20 = Mild Steel 2mm AL20 = Aluminum 2mm

F - ,.Finish applied to the base material (GGP = Ghost Gray Polyester powder coated

(XXX) Special Suffix to indicate CUSTOM features or other variants etc

PROFESSIONAL QUALITY IN COMPUTER & ELECTRONIC HARDWARE, COMPONENTS & MATERIALS