SP3P Applications

PSP3P Issue 3 PCB Laminate

|  | The C.A. Designs signal processor SP3P is a video converter for transforming video signals from SVGA or XGA PC graphics systems into composite analogue RGB video. It may be used to enable PC graphics adaptors to send their signals through conventional video switches, and / or down long cable runs, to remote monitor screens. It accepts incoming synchronisation pulses of any polarity. A composite video output is constructed from the input signals, modified by gating and time delays. The cable driving amplifiers provide optimum picture quality at the far end of long co-axial cable runs. Results depend on the originating signal and the cable quality but good pictures should be obtained up to: - 300 metres of RG59 coaxial cable with a 1024x768 pixel non-interlaced video signal - 200 metres of RG179 coaxial cable with a 1024x768 pixel non-interlaced video signal It is constructed as a PCI bus card for "in PC" applications (powered from the bus). Jumper Settings (factory defaults in BOLD type): Jumper; Notes; Details. J1; H sync duration; 1-2 = Variable 1 to 3uS by P1 3-4 = 1uS 5-6 = 2uS. J2; V sync delay; 1-2 = ~32uS 2-3 = ~64uS. J3; V sync duration; 1-2 = 64uS 2-3 = 32uS. J4; H sync delay; 1-2 = Variable 0.4 to 2.5uS by P2 3-4 = 0.5uS 5-6 = 1uS. J5; H sync phasing; 1-2 = Advanced, variable by P3 3-4 = Direct 5-6 = Delayed. J6; Input signal format; 1-2 = VGA / SVGA / XGA 2-3 = RGBS. J11; Red input signal termination; 1-2 = Terminated 75 ohm 2-3 = Unterminated. J12; Green input signal termination; 1-2 = Terminated 75 ohm 2-3 = Unterminated. |  |
| --- | --- | --- |

|  | Jumper Settings (Continued): Jumper; Notes; Details. J13; Input pin 8; 1-2 = Open 2-3 = Ground. J14; Input pin 7; 1-2 = Open 2-3 = Ground. J15; Blue input signal termination; 1-2 = Terminated 75 ohm 2-3 = Unterminated. Potentiometer settings: Potentiometer; Notes; Details. P1; Variable H sync duration; When selected. P2; Variable H sync delay; When selected. P3; Variable H sync advance; When selected. P4; Variable gain; Preset for 0.7v peak white video into 75 ohms. Cable Compensation – Jumpers: - J7, J8, J9 and J10 must all be set to the same setting - All OFF = switched compensation for up to 200 metres RG59 cable (jumper may be 'parked' on pin 1) - 1-2 = switched compensation for up to 200 metres RG179 cable - 2-3 = switched compensation for up to 300 metres RG59 cable Cable Compensation - Switches: - SW1, SW2, SW3 and SW4 must all be set to the same setting (only one switch per bank may be ON) - All OFF = No compensation - 1 ON = Least compensation (approx. 0.25x maximum length) - 2 ON = Intermediate compensation (approx. 0.5x maximum length) - 3 ON = Intermediate compensation (approx. 0.75x maximum length) - 4 ON = Maximum compensation - Choose setting that gives best 'on-screen' results - equal intensity horizontal and vertical lines with minimum shading or bright-up after blocks of white or black. INPUT SIGNALS. ## Signal Type J6 = 1-2; VGA, SVGA or XGA Analogue RGB video, TTL H &V (+ or -) sync.. ## Signal Type ## J6 = 2-3; Analogue RGB video, compsite HV TTL sync. (from SK1 pin 4). ## Connector; 9-way ‘D’ socket Pin 1 = Red Pin 2 = Green Pin 3 = Blue Pin 4 = H or HV sync Pin 5 = V sync or not used. |  |
| --- | --- | --- |

|  | OUTPUT SIGNALS. ## Signal Type; Analogue RGB (3 channels: red, green and blue) Sync mixed on green Positive video amplitude: 0.7v at remote end of cable (into 75 ohms) Negative sync amplitude: 0.3v at remote end of cable (into 75 ohms). ## Connector; 3 @ BNC sockets. CABLE COMPENSATION. ## Description; Matched to RG59U coax for lengths of 0, 75, 150, 225 and 300 metres Matched to RG179U coax for lengths of 0, 50, 100, 150 and 200 metres. POWER. ## Description; +12v DC @ 130mA max. Supplied via PC bus pins. SP3P – Jumper, Potentiometer and Switch Board Positions |  |
| --- | --- | --- |