REPORT	ON	KINGS	CRIMP

30th August, 1976.

NOTE: For restricted distribution.

Introduction:

CONFIDENTIAL

As a result of Sub-Contract works recently undertaken by the writer this report contains observations concerning the use of KINGS Crimp connectors and tooling.

Materials

2500m surplus (used) BICC type T3141, T3205 in random lengths 10m through

150m.

110 Kings type KC 59-181 Crimp Plug.

100 Kings type KC19-115 Crimp Jack.

Tooling

Kings Type KTH-1000 Hand Crimp Tool

Kings Type KTH-2012 Crimp Dies for above. Western Electronics CX-1 Stripping Tool.

Method

1.	cable was inspected for damage and the useful lengths measured and prepared at each end to receive the appropriate Crimp Connector.

2.	Crimp connectors were fitted by the manufacturer's recommended method where possible otherwise were modified to fit properly.

Observations

The following observations were made concerning the KINGS Products and our comments relating to experience with SUHNER products is made alongside.

OBSERVATIONS

1.	Hexagonal crimp centre contact is inferior to square crimp.	(Lower strength)

2.		Cross Hole (Solder Hole) on KINGS Jack occurs at the edge of the centre contact crimp and frequently the contact fractures as it is crimped.

3. 	The plating on KING'S ferrules frequently cracks and peels off when it has been crimped.

4. 	KINGS contacts are not captivated and must be located in the connector body visually. On lower quality cables (and especially foam dielectric cables) there is freq­ uently a tendency for movement to occur axially between the outer sheath, braid, dielectric and on rare occasions the

centre co uctor. 	If no captivation ex­ hists	the centre contact may with­ draw into the connector body (resulting in high resistance contact or in extreme cases no contact at all) or protrude

beyond the connector and causing damage to the mating connector as well.

COMMENTS

SUHNER use square;	see crimp catalogue Page 1 - 2.

SUHNER use longer crimp which does not effect solder hole. (See also Notes 7, 8, 26.)

We have never experienced this with SUHNER which exhibits better plating quality.

SUHNER Crimp connectors have either partial or full 2 way captivation. See Crimp Catalogue Page 1, Fig. lb thus ensuring perfect positioning

of the contact by feel before crimping and permanent location except under extreme tension which normally will destroy the cable before the connector.

4.(Cont)	The flexing, bending and temperature cycling which occurs in most practical situations causes movement between the cables parts and when centre contacts

are not captivated these contacts move

in sympathy to the cable.	During the

crimping operation a small amount of movement occurs due to the position and nature of the cable sheath.	Unless this movement is allowed for in the case of

the non-captive contact then wide variance can occur in the placement of centre con­ tacts in the connector body.

5. 		KINGS centre contacts are	silver plated and subject to tarnishing and short mechanical/electrical life.

6. 	KINGS cable assembly details must be found by reference to their catalogue and are very poorly set out.

7.	The KINGS crimp die produces a very narrow crimp on the centre contact which results in very localised and deep deformation with poor physical strength and VSWR compared to a longer square

form of crimp.

8. 	The portion of inner conductor which enters the centre contact hole is very short in the KINGS type.	This makes for difficult insertion of the inner conductor

since there is great tendency for the strands

of cable to fray instead of entering the con­ tact hole and in the case of the solid or single strand conductor (as with the stranded) there is a tendency for the contact to fall

off before or during the presentation of the assembly up to the crimp tool.

Sa.	No noticeable chamfer or lead in is provided on the KINGS centre contacts and they are therefore very difficult to fit onto the centre conductors particularly of stranded cables.

9. 	Some variance was detected in the size of

the centre contact hole in the KINGS connec­

tors.	Where this hole was too small to get the conductor inside it was necessary to drill contact out to appropriate size.

10.	Internal burrs from the cross-hole	drilling frequently restricted the proper insertion

of the cable conductor until the KINGS connec­

tor was drilled out to remove burr.

COMMENTS

SUHNER contacts are Gold Alloy plated, non tarnishing with

good electrical/mechanical life.

SUHNER supply quantities of individual instruction sheets with each box of connectors, extras on request.

SUHNER use longer (or wider) crimp for centre contact exhibit­ ing strength much in excess of

mil-C-39012 requirement while maintaining low VSWR.

SUHNER offer 4-5mm of inner conductor up into the centre contact and this minimises risk of contact falling off before crimp is effected,	Also with longer bared conductor strands can be twisted together into compact

bunch before offering up to con­

tact hole.

SUHNER have a useful chamfer for the inner contact hole which simplifies fitting.

Although on 1 or 2 occasions in over 5000 connectors a wrong

size of contact has been found in suhner packet, the spare contacts provided offer immediate solution to this problem.

SUHNER Connectors are burr free due to sharp drills and carefully devised drilling technique.

OBSERVATIONS

11.	No spare Contacts -or Crimp Ferrules are provided with KINGS connectors. The loss or destruction of either of these items accidently, means the total loss of value of the KINGS connectors since connector bodies

are rarely lost without the loss of these smaller parts.

12.	KINGS	Tools	are not provided

with any release mechanism.	If the

tool becomes fouled for any reason e.g.

1, 	Incorrect die

2. 	Incorrect connector part

3. 	Tool slips along from proper crimp position.

4. 	Foreign matter becomes lodged

between dies etc.

then it is necessary to destroy the connector or parts or to drive the

tool beyond the designed crimp pressure and the tool may give because the elastic limit or yield point has been exceeded.	When this permanent damage occurs proper crimps are no longer possible and the tool body (frame)

must be withdrawn from service.

13. 	KINGS tool KTH-1000 has quick exchange dies, however the mechanism by which this is achieved is not always reliable and the dies tend to become withdrawn from the tool body, fouling the connector.

14.	KINGS do not provide a cable cutting insert	which is a convenient aid for proper cutting of coaxial cables especially for larger sizes.

15...	KINGS connector packaging does not include the type number of connectors or assembly instruction reference on the airtight package.	The Type No.

is stamped on the connector body but

is not easily read through the package printing.	Connector impedance is not shown on KINGS connectors or packaging.

16. 		KINGS tooling (which is actually made by Buchanan) exhibits low quality design and materials.	The tool which we used had

not received much use yet showed need

for maintenance and low life expectancy.

17. 	The KINGS crimp spigot is split and when crimped may compress cable di-electric causing significant mismatch at UHF.

COMMENTS

SUHNER provide 2 spare contacts plus 2 spare ferrules with each box of connectors.	Extra contact and ferrules can be purchased for SUHNER connectors and therefore

connector bodies can be recovered.

SUHNER Tools incorporate a release catch and straining the tool can be avoided with

minimal effort.

SUHNER interchangeable tools have the dies secured by a Allen head grub screw.	An exchange system

on the small SUHNER hand tool

would be a desirable feature pro­

vided it does not suffer from the shortcomings of the KINGS tool.

SUHNER provide a cutting insert with the larger tools.

SUHNER provide both Part No. and Assembly reference of the air tight package.	It would be an advantage if basic cable pre­ paration could also be included on this package printing.

mpedance is integral with Pt. No.

and package coding.

SUHNER Tooling is of superior design and construction.	our tool is still giving perfect crimps after 5000+ connectors and we expect a further 10000+ before repair.

SUHNER spigot is solid.

3.

OBSERVATIONS

COMMENTS

18.

19.

20.

21.

22.

23.

24.

25.

KINGS KC 59-181 and KC 19-115 are 50 connectors and will introduce a mis­ match into 75 ohm transmission lines.

The PTFE dielectric insulation in some KINGS connectors is loose in the connector body indicating poor machining tolerances or lack of quality control.

We understand that KINGS use Nickel alloy plating which being magnetic produces undesirable effects on the RF propagation.

PTFE and other machining swarf has been observed in some KINGS connectors which may result in mating problems if not removed before assembly onto cables.

The location of the crimp ferrule before crimping is required to be determined by guess since no shoulder exists against which the ferrule and crimp tool can abut before crimping.

To reset the handle and ratchet on the KINGS tool it is required to open the handles to a width which is too wide to comfortably grip with one hand and two operations or use of both hands is required unless the operator has very large hands.

The moving lever of the handle of KINGS tool moves on the wrong side of the tool body creating an unnatural motion and poor tool control during crimping.	The tool handles are also the wrong shape for comfortable use.

KINGS ferrules and contacts are made either

SUHNER make 75 ohm connectors for 75 ohm coaxial cables.

We have not experienced this with SUHNER.

SUCOPLATE contains no magnetic alloy and is produced by world leader electroplater.

We have never observed any swarf in SUHNER connectors.

SUHNER have convenient shoulder to abut ferrule and tool steady before and during crimp.

SUHNER large tool has similar problems but since longer handles are used less effort is required.

SUHNER small tool has comfortable grips and is easily used because of natural motion of levers.

large tool can be bench mounted.

SUHNER use special annealed Beril­

from "Hard Brass" or some hard (not annealed) ium, Copper, Gold plated entre co1

26.

27.

28.

alloy and are therefore very brittle result­

ing in poor crimp junction with cable.

The KINGS tool weighs 800 grms with dies fitted and is tiresome to use.

KINGS dies must be removed from the tool body before tool can be replaced into storage box.

KINGS do not provide any device for bench mounting when large quantities of connectors are required to be fitted.

tacts, ferrules are of soft annealE

copper.	Work hardening occurs upo1 crimping.

Equivalent SUHNER tool weighs only 560 g.

SUHNER tools fit boxes with dies in the tool body except cutting insert.

SUHNER have bench mounting plate for large hand tool only.

29.

No stress relief sleeves or colour coding devices are made by KINGS for their connec­ tors.

SUHNER make coloured taper sleeves, caps and washers.

30.

KINGS require different tools for double screened cables compared to single screen types therefore more tools are required for the same range of cable types.

SUHNER tools crimp both double and single screen types since wall thickness of ferrule varies.

OBSERVATIONS	COMMENTS

31.	KINGS Crimp range includes:-	INC, TNC N, C, UHF, MHV, SHV, SMA.

SUHNER have most comprehensive range of Crimp Connectors for BNC, TNC, N, C, UHF, SMA, SMC,

SMB, SMS, H4, M, MCX, QLA, SHVetc.

3.2 	With the KINGS tool the centre contact can be crimped at any point along the body of the centre contact and unless

special care is taken to avoid misplacing the crimp a faulty joint can occur.

SUHNER contact is located in the die with a convenient jig or shoulder provided to ensure correct position of crimp.

CONCLUSIONS:

If the above observations are typical of KINGS the SUHNER Crimp technology is highly advanced and offers the most satisfactory results with least Operator skill or fatigue.

Longer and more reliable life can be expected from the SUHNER tooling and connectors in a wider range of situations.

A. W. DEVERELL

30/8/76.