ANDREW

INSTALLATION INSTRUCTIONS

CONNECTORS FOR FSJ1-50

SUPER FLEXIBLE HELIAX® COAXIAL CABLE

BULLETIN 37224B

| , | · 'iltPe Nwnber f | ---·- ' Tetmlhatlon | Connector | Fig.1 A Dim. in. (mm) | Fig.1B Dim. in. (mm) | Fig.4C Dim. in. (mm) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 41SP 41SW 40622 41SWT | Type UHF Plug Type N Plug Type BNC Plug Type TNC Plug | S0-239A UG-23 UG-89 TNC Jack | 1-19/32 (40) 29/32 (23) 3/4 (19) 11/16 (17) | 11/16 ( 17) 13/32 (10) 15/64 (6) 13/64 (5) | - - 39/64 ( 15) 9/32 (7) 9/32 (7) |

TOOLS AND  MATERIALS REQUIRED FOR ASSEMBLY

Garnet cloth

Knife

Wire brush

Scale, 6 in. (150 mm) Soft solder (rosin core) Wrenches (2) 7/16 in.

Soldering iron  (50 watt max.) Solvent,comothene, vythene or other non-flammable cleaning fluid

READ THE INSTRUCTIONS THOROUGHLY BEFORE  ASSEMBLY

STEP 1.    Cut cable as shown in Figure 1. Cable end must be square.   New cut can be made with sharp knife   in sawing motion.  Use straight-edged piece of paper around cable to  guide  jacketing  cut.     Clean outer conductor  with   solvent.    Cut off out.er con ductor   and  foam  with knife.   Use knife to remove burrs. 	Use wire brush to  remove  copper  particles from  foam.    Continue with Steps 2 and 3 for Type

41SP connector.     For   Types 41SW, 40622, and

41SWT connectors, read soldering techniques in Step

2 and proceed with Steps 4 and 5.

A 	-

FIGURE 1

STEP 2.   Slide UHF coupling nut onto cable with knurled end toward cable end.   Add adaptor until flush with end of outer conductor.   See Figure 2. Soft solder adapter to outer conductor. Use soldering iron  or  gun  and rosin  core  solder.    Avoid excessive heat.   Keep inner conductor centered.  Apply heat to adaptor to draw solder into hole.  Wipe away excess solder and cool connection with damp cloth.   Clean connection with   garnet cloth.   Do not use emery cloth   or steel wool.

INNER CONNECTOR

FIGURE 2

SOFT SOLDER

ADAPTOR

COUPLING NUT

Andrew Corporation 10500 W. 153rd St.    Andrew Antennas 	Andrew Antenas Umitada

Orland Park, IL  U.S.A.  60462	Lochgelly, Fife, Great Britain 	Sorocaba, SP. Brazil

Antennas  Andrew  S.A.R.L	Andrew  GmbH

Buc, France 	Munich. West Germany

Andrew Antenna Company  ltd. Whitby, Ontario, Canada

Andrew Antennas 	Antenas de Transmisi6n S.A de C.V 	Andrew  Corporation

Campbellfield, Victoria, Australia	Mexico, D.F. Mexico 	Tokyo, Japan

Andrew  S.R.L Milan, Italy

6/83

PRINTED

IN U.S.A.

STEP 3.   Screw UHF   inner connector firmly onto adaptor.     Soft   solder connector to adaptor and to inner conductor as shown in Figure 3.  Clean connec tion.  Screw coupling nut bnto inner connector until nut spins freely.

STEP 4.   Slide  Type  N, BNC  or  TNC  adaptor  onto cable  until flush  with end of  outer  conductor.  See Figure 4.    Soft solder adaptor to outer conductor. Slip inner connector onto   inner conductor until it bottoms against  inner  conductor.    Check reference "C" dimension  in table.  Soft  solder  inner connector to inner conductor.

INNER CONNECTOR

SOFT SOLDER

FIGURE 3

INNER

CONNECTOR 		SOFT SOLDER

COUPLING NUT

ADAPTOR

ADAPTOR

FIGURE 4

STEP 5.   Screw Type  N, BNC, or TNC coupling nut onto  adaptor  as shown in  Figure  5.   Tighten connec tion  with  wrenches.   Turn  coupling nut  only; do not turn  adaptor.   Correct  gap between coupling nut and adaptor   after   tightening  should   be  approximately

3/32 in. (2 mm).

COUPLING NUT

FIGURE 5