TECHNICAL	DATA

Section 3

TECHNICAL	DATA	& CIRCUIT DESCRIPTION

GENERAL SPECIFICATION

Application

General-purpose	solid-state		VHF receiver   with versatile	 frequency   control   suitable	for point- to-point	communications,	search  endsurvell!- ance,	monitoring   and  laboratory	use.

Reception Modes

AM -  FM -  CW -  PULSE Frequency Coverage

Models  1990R/1-X&	1990R/1-S::25-235MHz

in fi ve ranges.

) Models  1990R/2-X	& 1990R/2-S::25-500MHz

.			in  seven  ranges. Models  1990R/3-X	&  1990R/3-S::   Identical	to

1990R/2  butwith	the following  additional	  out- put  facilities.	(a) 5MHz  Synchronizer	Stan- dard   (b)  FM Audio.

Suffix  identity	'X'  : crystal  facility   (ten  chan- nels)	IS' : integral   synchronizer.

Intermediate	Frequency

21.4MHz.

FiIter Comple ment

Standard  bandwidths   : 250kHz  + 30kHz. Options:	'A'  250kHz  + 30kHz  +	15kHz

IB'  250kHz  + 30kHz  + 7.5kHz

'C'  250kHz  + 15kHz  + 7.5kHz

'01  250kHz  + 15kHz

'E'  250kHz   + 7.5kHz

Specify	30kHz  filter   b/w  for 50kHz  chann- elling.	 15kHz	25kHz

7.5kHz	12.5kHz

NB:  Use of crystal   facility	or synchronizer	is recommended	when  using  7. 5kHz  bandwidth.

Aerial	Inputs

50/75Q   unbalanced.	Additional	input  avai- lable  direct   to  bal.   mixer  for Wideband  app- lications.

Output	Facilities

Separate	 'wide'   and  'narrow'	IF outputs,   pos- itive  video,	negative	video,	external	loud- speaker,	headset   and  6000  lines  (bal.   or un- bal.  ).

Environmental

-10 o	to +50 0	(-20 0	to +70 0	storage). Calibration	Accuracy

C	C	C	C

Within   .5%   up  to  235MHz   and	within	1% from 235-500MHz   without  use of built-in   cal- ibrator   (Cursor  Adjuster   at  mid-position).

Calibrator	provides  1MHz and  1OMHz markers for precise   frequency	setting.

Power  Supplies

AC  : : 1OO/130V or 200/260V	(40-60Hz).

DC  : : ·12V with  negative	earth.

Consumption	(AC/12V)	:  RECEIVER -   15W/ lOW RECEIVERC OMPLETEWITH SYNCHRO- N IZER  -35W/30W.

Dimensions	and   Weight	(with  Synchronizer) Rack-mounti  n9, style:

Panel:	483mm x  133mm (19in.   x 5.25in).

Rack  intrusion:   411mm (16. 1875in). Weight:	18.4kg   (40.5Ib).

Bench-mounting	style: Width:	502mm  (l9.  75in).

Height:		(including	feed)  :  164mm (6.5in.). Dept:	457mm  (l8in.).

Weight:	21.3kg.	(47Ib.)

Page  19

TECHNICAL	DATA

RANGE

1

2

3

4

5

6

7

PERFORMANCE	SUMMARY

FREQUENCY COVERAGE

FREQUENCY	.

25MHz -	39MHz

39MHz-	60MHz

60MHz -95MHz

87MHz .. 140MHz

140MHz .. 235MHz

23SMHz .. 365MHz

360MHz .. SOOMHz:

I F  Rejection

Typical data: not to be interpreted as a test specification.

| Sensitivity (1OdBS+N/N with standard o/p) MODE | BW | 25-140 | 140-235 | 235-500 |
| --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MH~ |

Greater than 60dB above 35MHz.

Stability

Free running oscillotor ; 1 part in la5_~C. Crystal oscillator	: 1 part in liP I!.e.

Synchronizer	: O.S part in 107~C.  *

*AM

**FM

CW

30kHz

2S0kHz

30kHz

l~V

s..V

~V

(*) -10°C to +50oC.

(*) 30% mod at 1kHz. (**) 22.SkHzdeviation.

Noise Factor

Typically 4dB and of the order 8dB at any frequency: better than 10dB direct to Mixer.

Image Rejection

AFC Hold

At least 1% of signal frequency (0p"rative in free-run mode  only).

Selectivity

Dependent  on fil"r	complement (see page 19) WDE :	25OkHzat ..3dB (ve filter)

NARROW:	PASSBAND	STOPBAND

Greater than SOdBto 200MHz

40dB 23SISOOMHz.

30kHz (+	15kHz)

15kHz (+ 7. 5kHz)

7.SkHz (t3.75kHz)

+501cHz

+25kHz

+12.SkHz

Page 20

Dynamic  Range

Video Response

TECHNICAL	DATA

40dB (with  AGe   disabled)  •   Taken  for 5%

total  distortion   with  60%  modulation   at

1kHz.

AGC  Characteristics

less  than  10dB change   in output  for 80dB

increase   in  input  from 3..N.

FM  Deviation

li near  acceptance	to 75kH z ,

Audio  Output

) Ext.  loudspeaker	(30)	1.fNI at  10%

distortion.

line   (6000  with  CT)	20mW at  5%

distorti on. Headset	 low/medium-Z. Response	Within   3dB,

200Hz-8kHz.

Video Outputs

Separate   +ve and  -ve  outputs  of  lV p-p  into

500.

level   within  +3dB from 20Hz  to  250kHz.

RF Oscillator   Output

50-150mV   into  5OQ.

5MHz  Std. Output

(3S version  only)  350mV into  500.

FM Audio  Output

(3S version  only)  approx  900mV  into  10kQ.

IF  Outputs	(quoted  for  lOI-'V signal) Separate   wide  and  narrow-band	outputs

matched   to 5OQ.

low-level	wideband   output:		151-'V Narrow-band	output  *	: 50mV

(1) B/W set  by receiver	IF  selectivity.

Muting

Threshold  adjustable	to  31-'Vcarrier.

Two MUTING  options  are  available	depen- dent  on Module  fitted:-

| OPTION A:: | 23kHz NOMINAL:: | lP3396 |
| --- | --- | --- |
| OPTION B:: | 15kHz NOMINAL:: | lP3396/1 |

Page  20A